# Packet Tracer - Examine the ARP Table

# Addressing Table

| Device | Interface | MAC Address | Switch Interface |
| --- | --- | --- | --- |
| Router0 | Gg0/0 | 0001.6458.2501 | G0/1 |
| Router0 | S0/0/0 | N/A | N/A |
| Router1 | G0/0 | 00E0.F7B1.8901 | G0/1 |
| Router1 | S0/0/0 | N/A | N/A |
| 10.10.10.2 | Wireless | 0060.2F84.4AB6 | F0/2 |
| 10.10.10.3 | Wireless | 0060.4706.572B | F0/2 |
| 172.16.31.2 | F0 | 000C.85CC.1DA7 | F0/1 |
| 172.16.31.3 | F0 | 0060.7036.2849 | F0/2 |
| 172.16.31.4 | G0 | 0002.1640.8D75 | F0/3 |

- Blank Line, No additional information

# Objectives

Part 1: Examine an ARP Request

Part 2: Examine a Switch MAC Address Table

Part 3: Examine the ARP Process in Remote Communications

# Background

This activity is optimized for viewing PDUs. The devices are already configured. You will gather PDU information in simulation mode and answer a series of questions about the data you collect.

# Instructions

## Examine an ARP Request

### Generate ARP requests by pinging 172.16.31.3 from 172.16.31.2.

Open a command prompt

Click 172.16.31.2 and open the Command Prompt.

Enter the arp -d command to clear the ARP table.

Close a command prompt

Enter Simulation mode and enter the command ping 172.16.31.3. Two PDUs will be generated. The ping command cannot complete the ICMP packet without knowing the MAC address of the destination. So the computer sends an ARP broadcast frame to find the MAC address of the destination.

Click Capture/Forward once. The ARP PDU moves Switch1 while the ICMP PDU disappears, waiting for the ARP reply. Open the PDU and record the destination MAC address.

#### Question:

Is this address listed in the table above?

Click Capture/Forward to move the PDU to the next device.

#### Question:

How many copies of the PDU did Switch1 make?

3

What is the IP address of the device that accepted the PDU?

172.16.31.3

Open the PDU and examine Layer 2.

#### Question:

What happened to the source and destination MAC addresses?

Source becomes Destination and Destination to Source

Click Capture/Forward until the PDU returns to 172.16.31.2.

#### Question:

How many copies of the PDU did the switch make during the ARP reply?

1

### Examine the ARP table.

Note that the ICMP packet reappears. Open the PDU and examine the MAC addresses.

#### Question:

Do the MAC addresses of the source and destination align with their IP addresses?

Switch back to Realtime and the ping completes.

Click 172.16.31.2 and enter the arp –a command.

#### Question:

To what IP address does the MAC address entry correspond?

172.16.31.3

In general, when does an end device issue an ARP request?

When it doesn’t know the destination MAC address

## Examine a Switch MAC Address Table

### Generate additional traffic to populate the switch MAC address table.

Open a command prompt

From 172.16.31.2, enter the ping 172.16.31.4 command.

Click 10.10.10.2 and open the Command Prompt.

Enter the ping 10.10.10.3 command.

#### Question:

How many replies were sent and received? Close a command prompt

4

### Examine the MAC address table on the switches.

Click Switch1and then the CLI tab. Enter the show mac-address-table command.

#### Question:

Do the entries correspond to those in the table above?

yes

Click Switch0, then the CLI tab. Enter the show mac-address-table command.

#### Questions:

Do the entries correspond to those in the table above?

yes

Why are two MAC addresses associated with one port?

The Switches are all directing traffic to a single point through a single router

## Examine the ARP Process in Remote Communications

### Generate traffic to produce ARP traffic.

Open a command prompt

Click 172.16.31.2 and open the Command Prompt.

Enter the ping 10.10.10.1 command.

Type arp –a.

#### Question:

What is the IP address of the new ARP table entry?

1.72.16.31.1

Enter arp -d to clear the ARP table and switch to Simulation mode.

Repeat the ping to 10.10.10.1.

#### Question:

How many PDUs appear?

2

Close a command prompt

Click Capture/Forward. Click the PDU that is now at Switch1.

#### Question:

What is the target destination IP destination address of the ARP request?

The destination IP address is not 172.16.31.1

#### Question:

Why?

The gateway address is the default address the ARP request will be sent to and forwarded from if the router knows the IP Address

### Examine the ARP table on Router1.

Switch to Realtime mode. Click Router1 and then the CLI tab.

Enter privileged EXEC mode and then the show mac-address-table command.

#### Question:

How many MAC addresses are in the table? Why?

nothing

Enter the show arp command.

#### Questions:

Is there an entry for 172.16.31.2?

yes

What happens to the first ping in a situation where the router responds to the ARP request?

It ignores it

End of document